PROJECT REFERENCE GUIDE

AgroSense

Smart irrigation monitoring and control platform

Current implementation guide · Version 0.1.0 · 7 September 2026

Purpose: This document is the single project reference for the current AgroSense product scope, architecture, setup, API contracts, device integration, and implementation status.

# Start here

Run the backend and frontend in separate terminals. The backend can start without PostgreSQL or MQTT; it uses seeded in-memory telemetry until those services are configured.

Terminal 1 · backend

cd backend

cp .env.example .env

npm install

npm run dev

Terminal 2 · frontend

cd frontend

cp .env.example .env

npm install

npm run dev

Open the dashboard at http://localhost:5173. The Express API runs at http://localhost:4000.

Production build: Use npm run build in both backend and frontend directories. Start the compiled backend with npm start.

# Project at a glance

| Area | Current choice |
| --- | --- |
| Product | IoT-enabled smart irrigation dashboard for farm monitoring and control |
| Frontend | React, TypeScript, Vite, local shadcn-style UI primitives, Lucide icons |
| Backend | Node.js, Express, TypeScript, Socket.IO, MQTT.js, Zod |
| Persistence | PostgreSQL through pg; in-memory fallback for development |
| Device | ESP32 sensor/controller sending telemetry over Wi-Fi |
| Telemetry protocol | MQTT topic agrosense/{device_id}/telemetry |
| Live UI protocol | Socket.IO events telemetry:update and dashboard:summary |
| Current status | Dashboard shell, live summary, validation, MQTT gateway, and PostgreSQL write-through are implemented |

# Requirements from the supplied structure document

The attached structure document is treated as the product/design reference. Its page descriptions are requirements for the AgroSense application, not additional instructions that override the user's request.

| Module | Scope |
| --- | --- |
| Login | User login, password visibility, remember-me option, forgot-password flow, and optional Google login treatment. |
| Dashboard | Temperature, humidity, water level, rain status, flow rate, all-zone soil moisture, pump status, water usage, and system status. |
| Live Sensor Data | Real-time readings, zone-wise moisture, live moisture trends, current sensor status, tank status, and rain status. |
| Zones | Independent monitoring and management of Zone 1, Zone 2, and Zone 3, including crops, pump state, trends, comparison, and zone irrigation details. |
| System Control | Automatic/manual modes, zone pump control, moisture thresholds, schedules, flow monitoring, and water-tank status. |
| Irrigation History | Past events with date/time, zone, moisture before/after, duration, water used, pump status, filters, and charts. |
| Plant Analysis | Plant health, growth stage, recommended moisture, environmental conditions, plant response, water fingerprint, and recommendations. |
| Alerts | Low moisture, high temperature, low water, rainfall, pump, offline, system alerts, history, and notification settings. |
| Settings | Device ID, Wi-Fi settings, thresholds, notifications, system preferences, data management, and save settings. |

# Architecture

The ESP32 communicates over Wi-Fi. MQTT is the recommended device-to-backend transport because it is lightweight, reconnectable, and suitable for continuous sensor messages.

ESP32 sensors / pumps

| Wi-Fi + MQTT

v

Express backend

|-- MQTT ingestion and validation

|-- PostgreSQL write-through

|-- Alert and irrigation services

|-- Socket.IO live updates

v

React dashboard

## Repository structure

backend/

src/config              Environment parsing

src/middleware          Express error handling

src/routes              Versioned API routes

src/services            Telemetry store and MQTT service

src/types               Shared telemetry types

src/validation          Zod schemas

prisma/schema.prisma    Reference application data model

frontend/

src/components/ui       Local shadcn-style primitives

src/App.tsx             Dashboard and navigation shell

src/api.ts              REST client

src/styles.css          AgroSense visual system and layout

components.json         shadcn component configuration

# Frontend implementation

The frontend currently provides the AgroSense dashboard shell and uses local shadcn-style components so the UI is editable and consistent within the repository.

Button: navigation, panel actions, and action variants

Card: metric cards, zone cards, and dashboard panels

Badge: device, connection, and pump status

Avatar: admin user chip

Progress: zone soil-moisture levels

Separator: topbar structure

Lucide icons: consistent navigation and metric icon alignment

Implemented dashboard content includes connection status, five system metrics, soil moisture by zone, water-usage visualization, moisture trend, pump donut, system status, and a branded sustainability banner.

# Environment configuration

| Variable | Default / example | Purpose |
| --- | --- | --- |
| PORT | 4000 | Express API port |
| CLIENT_ORIGIN | http://localhost:5173 | Allowed frontend origin |
| DATABASE_URL | empty | PostgreSQL connection string; empty enables in-memory fallback |
| MQTT_ENABLED | false | Enable ESP32 MQTT ingestion |
| MQTT_URL | mqtt://localhost:1883 | MQTT broker address |
| MQTT_USERNAME / MQTT_PASSWORD | empty | Optional broker credentials |
| MQTT_TOPIC_PREFIX | agrosense | Topic namespace prefix |
| VITE_API_URL | http://localhost:4000/api/v1 | Frontend REST base URL |
| VITE_SOCKET_URL | http://localhost:4000 | Frontend Socket.IO origin |

# Telemetry contract

The backend accepts a payload with a device ID, system readings, and one or more zone readings. The timestamp is optional; the backend assigns the current time when omitted.

{

"device_id": "AGROSENSE01",

"timestamp": "2026-09-07T10:30:00Z",

"system": {

"temperature": 29.5,

"humidity": 64,

"water_level": 78,

"rain": false,

"flow_rate": 1.8

},

"zones": [

{ "zone_id": 1, "soil_moisture": 28, "pump_status": true },

{ "zone_id": 2, "soil_moisture": 52, "pump_status": false },

{ "zone_id": 3, "soil_moisture": 21, "pump_status": true }

]

}

# Express API

| Method | Endpoint | Purpose |
| --- | --- | --- |
| GET | /api/v1/health | Service health check |
| GET | /api/v1/dashboard/summary | Latest dashboard summary; optional device_id query parameter |
| GET | /api/v1/devices/:deviceId/latest | Latest raw telemetry for a device |
| POST | /api/v1/devices/:deviceId/telemetry | Validated HTTP telemetry simulator/fallback endpoint |

Example telemetry test:

curl -X POST http://localhost:4000/api/v1/devices/AGROSENSE01/telemetry \

-H 'content-type: application/json' \

--data @telemetry.json

# ESP32 and MQTT integration

Set MQTT_ENABLED=true in backend/.env when the broker is available. The backend subscribes to:

agrosense/{device_id}/telemetry

The ESP32 should publish validated JSON telemetry periodically and reconnect automatically after Wi-Fi loss. Future control commands should use separate command and acknowledgement topics:

agrosense/{device_id}/commands

agrosense/{device_id}/commands/ack

Safety requirement: The ESP32 should enforce local safety rules for low tank level, missing flow, maximum pump duration, and conflicting pump commands. It must not depend entirely on cloud connectivity for physical safety.

# Current implementation status

| Area | Status | Notes |
| --- | --- | --- |
| Express backend | Implemented | TypeScript, validation, health, dashboard summary, telemetry endpoint |
| MQTT gateway | Implemented | Disabled by default; validates incoming ESP32 payloads and broadcasts live updates |
| PostgreSQL write-through | Implemented | Creates telemetry tables when DATABASE_URL is configured; falls back safely otherwise |
| React dashboard | Implemented | Dashboard shell, live connection state, cards, charts, zones, status panels |
| Authentication | Planned | Login UI is in the product scope; protected auth routes are not yet implemented |
| Pump control | Planned | Safety-oriented API and ESP32 command acknowledgement are next |
| History, alerts, plant analysis, settings | Planned | Navigation placeholders exist; domain modules remain to be built |
| Real hardware validation | Pending | Use an MQTT simulator before connecting physical pumps |

# Recommended build sequence

Confirm local setup with the seeded dashboard and HTTP telemetry endpoint.

Run an MQTT broker and replace HTTP simulator messages with ESP32 telemetry.

Connect PostgreSQL and verify telemetry hydration after backend restart.

Implement authentication and protected routes.

Build zones and safe manual/automatic irrigation control.

Add irrigation history and CSV export.

Add the alert rules engine and notification preferences.

Build plant analysis from rule-based insights, then consider predictive models after historical data exists.

Test device failure, stale data, low tank, rain, abnormal flow, and Wi-Fi loss scenarios.

Deploy with HTTPS, a managed PostgreSQL database, an MQTT broker, and application monitoring.

# Verification checklist

backend npm run typecheck passes

backend npm run build passes

frontend npm run typecheck passes

frontend npm run build passes

GET /api/v1/health returns status ok

POST telemetry accepts valid readings and updates the dashboard summary

Invalid telemetry returns HTTP 400

MQTT remains disabled until explicitly configured

PostgreSQL fallback keeps local development usable without a database

Frontend connects through REST and Socket.IO

# Troubleshooting

## Port already in use

The backend uses port 4000 and the frontend uses port 5173 by default. Stop the existing process or set PORT / Vite server configuration before starting another instance.

## Dashboard shows offline

Start the backend, confirm the API URL, and submit telemetry. The in-memory store marks a device offline when its latest reading is more than five minutes old.

## PostgreSQL is unavailable

Leave DATABASE_URL empty for local UI work. The backend logs the fallback and continues with seeded in-memory telemetry.

## MQTT is unavailable

Leave MQTT_ENABLED=false and use the HTTP telemetry endpoint while the broker or ESP32 is not ready.

# Source and scope note

The functional scope and sample data in this guide are consolidated from the supplied AgroSense structure document and the implementation currently present in this repository. Product screenshots are treated as visual direction; safety behavior, authentication policy, hardware wiring, notification channels, and plant-analysis formulas still require product decisions before production release.